L Series Led Street Light

High-quality aluminum alloy, with integrated shell, high hardness, not easy to break, good heat dissipation, greatly improving service life.

The lamp body is completely sealed and work in thunder and rain.

High-quality high-brightness LED light source, high luminous efficiency, high color rendering, no radiation

pollution, long service life.

No EMI, low harmonic distortion (THD), through the CE, ROSS, FCC, SAA, RCM certification requirements.

Working under the ambient temperature -30°to 55°

Overview

LED Source: Philips 3030/5050 LED

Input Vol: AC100-277V

SPD:10KV

CRI: >75Ra

IP Rate: IP66

Warranty: 5 years

Beam angle: Type 2 / Type 3

| Model | Housing Dimensions(mm) | Housing Net weight | Suitable for Wattage |
| --- | --- | --- | --- |
| L series -Mini | 200x555x105 | 2.5KGS | 30w,40w,50w,60w |
| L series -Medi | 233x615x100 | 3.1KGS | 75w,80w,100w |
| L series -Large | 260x675x100 | 3.8KGS | 120w,150w |
| L series -Max | 310x726x113 | 5.1KGS | 180w,200w240w |

Buckle:

Easy to disassemble

Mounting arm:

0-90° adjustable

PMMA Lens:

PC+Glasses

Power off switch:

Safe for installing and maintenance

Production dimensions

50W        100W         150W         200W

45mm	      52mm           62mm

Products Characteristics-1

| Model： | FL-LSH50W-L01 | FL-LSH100W-L02 | FL-LSH150W-L03 | FL-LSH200W-L04 |
| --- | --- | --- | --- | --- |
| Wattages: | 30w/40W/50W/60W | 70W/80W/100W | 120W/150W/160W | 180W/200W/240W |
| Led Source: | Philips-3030LED | Philips-3030LED | Philips-3030LED | Philips-3030LED |
| Led Qnty: | 96pcs | 144pcs | 192pcs | 288pcs |
| Working voltage： | AC100-277V | AC100-277V | AC100-277V | AC100-277V |
| Driver | Inventronics / Meanwell | Inventronics / Meanwell | Inventronics / Meanwell | Inventronics / Meanwell |
| Beam angle: | Type 2/3 | Type 2/3 | Type 2/3 | Type 2/3 |
| Lumen efficiency： | 120-130lm/w | 120-130lm/w | 120-130lm/w | 120-130lm/w |
| Power Factor: | >0.95 | >0.95 | >0.95 | >0.95 |
| Correlated Color Temperature： | 3000K-6500K | 3000K-6500K | 3000K-6500K | 3000K-6500K |
| Color rendering index： | ≥70 | ≥70 | ≥70 | ≥70 |
| IP Rating: | IP66 | IP66 | IP66 | IP66 |
| Material | Aluminum Alloy | Aluminum Alloy | Aluminum Alloy | Aluminum Alloy |
| Weight: | 2.8kgs | 3.9kgs | 4.8kgs | 6.2kgs |
| Warranty | 5 years | 5 years | 5 years | 5 years |

Products Electrical Paraments

|  |  |  |  | Nominal flux (lm) (*) | Power consumption (W) | Nominal efficacy (lm/W) | Luminaire output flu(lm) | Luminaire efficacy (lm/W) | Photometry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lamp | 3030 LEDs | Current (mA) | Line Current (A) | (A) Typical | Typical | Typical | Typical | Typical | KingBrite T3 |
| MINI | 96 | 770 | 0.14 | 4680 | 30W | 156 | 3840 | 128 | KingBrite T3 |
|  | 96 | 1000 | 0.19 | 6080 | 40W | 152 | 5000 | 125 |  |
|  | 96 | 1300 | 0.23 | 7300 | 50W | 146 | 6100 | 122 |  |
|  | 96 | 1550 | 0.27 | 7920 | 60W | 132 | 6480 | 108 |  |
| MEDI | 144 | 1800 | 0.32 | 10850 | 70W | 155 | 8890 | 127 | KingBrite T3 |
|  | 144 | 2050 | 0.36 | 12080 | 80W | 151 | 9920 | 124 |  |
|  | 144 | 2550 | 0.45 | 14800 | 100W | 148 | 12100 | 121 |  |
| LARGE | 192 | 3050 | 0.54 | 18720 | 120W | 156 | 15360 | 128 | KingBrite T3 |
|  | 192 | 3800 | 0.68 | 22800 | 150W | 152 | 18750 | 125 |  |
|  | 192 | 4050 | 0.72 | 23680 | 160W | 148 | 19360 | 121 |  |
| MAX | 288 | 4550 | 0.82 | 27900 | 180W | 155 | 22860 | 127 | KingBrite T3 |
|  | 288 | 5000 | 0.91 | 30200 | 200W | 151 | 24800 | 124 |  |
|  | 288 | 6000 | 1.09 | 29600 | 240W | 148 | 29040 | 121 |  |

Tolerance on LED flux is ± 7% and on total luminaire power ± 5%

(*) The nominal flux is an indicative LED flux @ Ts 85°C based on LED manufacturer’s data. The real flux output of the luminaire depends on

environmental conditions (e.g. temperature and pollution) and the optical efficiency of luminaire. The type of LED used is subject to change

due to the ongoing rapid progress taking place in LED technology

Products Characteristics-2

| Model： | FL-LSH50W-L01 | FL-LSH100W-L02 | FL-LSH150W-L03 | FL-LSH200W-L04 |
| --- | --- | --- | --- | --- |
| Wattages: | 30W/40W/50W/60 | 70W/80W/100W | 120W/150W/160W | 180W/200W/240W |
| Led Source: | Philips-5050LED | Philips-5050LED | Philips-5050LED | Philips-5050LED |
| Led Qnty: | 24pcs/36pcs | 54pcs | 72pcs | 120pcs |
| Working voltage： | AC100-277V | AC100-277V | AC100-277V | AC100-277V |
| Driver | Meanwell/inventronics | Meanwell/inventronics | Meanwell/inventronics | Meanwell/inventronics |
| Beam angle: | Type 2/3 | Type 2/3 | Type 2/3 | Type 2/3 |
| Lumen： | 7500lm | 15000lm | 22500lm | 30000lm |
| Power Factor: | >0.95 | >0.95 | >0.95 | >0.95 |
| Correlated Color Temperature： | 3000K-6500K | 3000K-6500K | 3000K-6500K | 3000K-6500K |
| Color rendering index： | ≥70 | ≥70 | ≥70 | ≥70 |
| IP Rating: | IP66 | IP66 | IP66 | IP66 |
| Material | Aluminum Alloy | Aluminum Alloy | Aluminum Alloy | Aluminum Alloy |
| Weight: | 2.8kgs | 3.9kgs | 4.8kgs | 6.2kgs |
| Warranty | 5 years | 5 years | 5 years | 5 years |

Products Electrical Paraments

|  |  |  |  | Nominal flux (lm) (*) | Power consumption (W) | Nominal efficacy (lm/W) | Luminaire output flu(lm) | Luminaire efficacy (lm/W) | Photometry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lamp | 5050 LEDs | Current (mA) | Line Current (A) | (A) Typical | Typical | Typical | Typical | Typical | KingBrite T3 |
| MINI | 24 | 600 | 0.14 | 5730 | 30W | 191 | 4680 | 156 | KingBrite T3 |
|  | 24 | 800 | 0.19 | 7520 | 40W | 188 | 6160 | 154 |  |
|  | 24 | 1000 | 0.23 | 8900 | 50W | 178 | 7300 | 146 |  |
|  | 36 | 1200 | 0.27 | 11460 | 60W | 191 | 9420 | 157 |  |
| MEDI | 54 | 1400 | 0.32 | 13510 | 70W | 193 | 11060 | 158 | KingBrite T3 |
|  | 54 | 1600 | 0.36 | 15040 | 80W | 188 | 12320 | 154 |  |
|  | 54 | 2000 | 0.45 | 18500 | 100W | 185 | 15200 | 152 |  |
| LARGE | 72 | 2400 | 0.54 | 22920 | 120W | 191 | 18840 | 157 | KingBrite T3 |
|  | 72 | 3000 | 0.68 | 28500 | 150W | 190 | 23400 | 156 |  |
|  | 72 | 3200 | 0.72 | 29440 | 160W | 184 | 24160 | 151 |  |
| MAX | 108 | 3600 | 0.82 | 33840 | 180W | 188 | 27720 | 154 | KingBrite T3 |
|  | 108 | 4000 | 0.91 | 37000 | 200W | 185 | 30400 | 152 |  |
|  | 108 | 4800 | 1.09 | 44160 | 240W | 184 | 36240 | 151 |  |

Tolerance on LED flux is ± 7% and on total luminaire power ± 5%

(*) The nominal flux is an indicative LED flux @ Ts 85°C based on LED manufacturer’s data. The real flux output of the luminaire depends on

environmental conditions (e.g. temperature and pollution) and the optical efficiency of luminaire. The type of LED used is subject to change

due to the ongoing rapid progress taking place in LED technology

Photometric distribution

Type II                             Type III

Dissamble illustration

LED Driver

SPD 10KV/20kv

Disconnection protection

Rubber gasket

Earth line

Aluminum alloy housing

Spigots:45/52/62mm optional

LED Beads

Tempered glass

Acrylic PMMA Lens

Driver box

Buckle

White nano reflector

Intelligent Dimming Control

The dimming photo controller JL-241/242/243 series is applicable to control any independent LED home/street lighting automatically in accordance with the ambient natural lighting level. This product is designed with microprocessor circuits with IR-filtered silicone sensor, a heavy duty surge arrester (MOV) is provided to protect the lamp from inrush current as well as impulse by lightening. Further, a preset 10 seconds time-delay prevents excessive operation due to spotlight or lightning during the night time. A heavy duty relay supports the work life of over 15,000 cycles, and offers extra long work life when an optional metal Armor is applied together. This product provides twist lock terminals and signal contacts meeting the requirements of ANSI C136.41-2013, as well as being listed by UL under the Standard for Plug-In, Locking Type Photo controls for Use with Area Lighting ANSI/UL773.

# Constant On/Off Dimming Standard program for LED output keeping the field lit with constant lighting level, in accordance with the natural sunlight level detected. When the natural lighting is higher than 110% of the rated level,live line will be disconnected to keep LED fixture as zero consumption Available with JL-241C, JL-242C and JL-243C.

| Items | Function (0~10V dimming) | Items | Function (DAU dimming) |
| --- | --- | --- | --- |
| JL-241C | Constant illumination ( Bright in the evening, off in the morning ) | JL-251C | Constant illumination ( Bright in the evening, off in the morning ) |
| JL-242C | Constant illumination, Midnight low brightness | JL-252C | Constant illumination, Midnight low brightness |
| JL-243C | Constant illumination, Midnight low brightness, Optical decay compensation | JL-253C | Constant illumination, Midnight low brightness, Optical decay compensation |
| JL-244C | Constant illumination, Midnight low brightness, Optical decay compensation, Mobile APP Control | JL-254C | Constant illumination, Midnight low brightness, Optical decay compensation, Mobile APP Control |

Breathing valve Optional：

Waterproof

Air circulation

Pressure balance

Driver brand optional：

Turefull

Meanwell

Philips

Inventronics

SPD optional：

SPD 10KV

SPD 20KV

City Smart lighting Control system

Zigbee wireless system

LoRa wireless system

- Installation Instructions

Attention:

- Be sure disconnect power before installing, don’t construction with electricity.

- Product working voltage is AC100~277V,50/60Hz，Please do not exceed the working voltage range.

- Input product line, brown is L ,blue is N, yellow-green is GND.

- Be sure wiring is closed and prevent leakage, when installing.

- The lamp can not be used in violation of any fire regulations.

- Package

| Item | FL-LSP50W-L01 | FL-LSP100W-L02 | FL-LSP150W-L03 | FL-LSP200W-L04 |
| --- | --- | --- | --- | --- |
| Package Qty | 1 PC | 1 PC | 1 PC | 1 PC |
| GW | 3.7 KGS | 4.8KGS | 5.8 KGS | 7.2KGS |
| Cartons (mm) | 630*230*150MM | 670*280*150MM | 730*300*150MM | 760*350*150MM |

Warranty Policy

All products are under 5-year warranty  since the date of luminaires were deliver to you.

1. Under warranty terms,if products are broken down under normal usage within warranty period, company will provide new lamp for instead of or provide FREE Maintenance service base on the proof-documents and invoice (copy) of the failed products.

2.  company will not provide paid maintenance service under following conditions even within warranty period.

a )Failure or damage caused by force majeure.

b) Failure or damage  caused by transportation or unloading

c) Failure or damage  caused by incorrectly usage or fail to comply with the manual and caution

d) Failure or damage  caused by demolition, repair or alternation without our authorization.

3.  company always reserves the right for final explanation of aforesaid terms